VELSANET

Structure-Native Network Architecture

Seoul Metropolitan 6G Network Infrastructure Deployment Plan

Seoul Metropolitan 6G Infrastructure Deployment Plan

— Including Velsanet 6G RU Design —

| Document No. | Velsanet-DEPLOY-SEO-01 (Rev.1) |
| --- | --- |
| Date | March 2026 |
| Prepared by | Velsanet Architecture Group / Velsanet Research Institute (Proposed) |
| Scope | Greater Seoul · Population approx. 10M · Area 605 km² |
| Reference Documents | WP #02, #03, #22 · Velsanet Architecture Compendium v1.0 (2026) |
| Key Revisions | RU/H6 count: fully recalculated based on 400K surveyed utility poles · O8 face structure applied |

# 1. Overview and Deployment Premises

This document defines the node counts (H6, RU, MOCT ribbon fiber, O8) and cable distances required to physically deploy the Velsanet Structure-Native 6G architecture across Seoul. Rev.1 adopts 400,000 surveyed utility poles as the deployment ceiling, fully recalculates RU and H6 counts, and derives accurate O8 quantities based on the O8 face structure (uplink 1–2 / downlink 1–2 / horizontal 3–4).

This architecture replaces multiple legacy networks (FTTx, mobile backhaul, metro rings, aggregation layers) with a unified optical infrastructure.

## 1.1 Seoul Baseline Data

| Item | Value | Source / Note |
| --- | --- | --- |
| Total Area | 605 km² | Seoul Metropolitan Government Official Statistics |
| Effective Coverage Area | 480 km² | Approx. 80% excl. mountains and parks |
| Population | approx. 10 million | As of 2024 |
| Population Density | approx. 16,000 /km² | Based on effective coverage area |
| Households | approx. 4.2 million | Avg. 2.4 persons/household |
| Office / Commercial Units | approx. 800,000 | SMEs and corporations combined |
| Mobile Subscriptions | approx. 12 million lines | 1.2 lines per person |
| Utility Poles (shared power/telecom) | approx. 400,000 | Seoul city survey — RU deployment ceiling |

※ RU cell radius assumed 300m for 6G mmWave. May be adjusted to 200–400m based on building density and radio obstacles.

※ The 400,000 utility poles are based on Seoul city survey and serve as the RU deployment ceiling (1 pole = 1 RU). This replaces the previous calculation of 312,500 (population ÷ 32 cores) with a pole-survey basis at 48 cores/RU.

# 2. RU (Radio Unit) Count Calculation

In the Velsanet Structure-Native RAN, each RU operates as a directional wireless access node mapped 1:1 to a physical optical core — not a conventional omnidirectional cell. Deployment count is capped by actual urban infrastructure (utility poles), not population capacity.

## 2.1 RU Count — 400K Utility Poles × Directional Antenna Basis

| Calculation Item | Formula / Value | Result |
| --- | --- | --- |
| Seoul Utility Poles (deployment ceiling) | Survey-based | 400,000 |
| RU Deployment Mode | Directional antenna RU 1–3 per pole (avg. 2.25) | Directional antenna · avg. 2.25/pole |
| Planned Deployment (poles × avg. RU) | 400,000 | 900,000 RU |
| Wireless Capacity Verification | 400,000 × 48 cores | 900,000 × 48 cores = 43.2M cores ≥ 12M subscriptions ✅ |

※ RUs are directional antennas installed 1–3 per pole. At design average 2.25/pole: 400,000 × 2.25 = 900,000 RUs. At 48 cores/RU: 43.2M total cores — 3.6× headroom over Seoul's 12M mobile subscriptions. Surplus cores serve IoT, V2X, high-density video, and PAI workloads.

※ Seoul's urban core (Jongno, Gangnam, Yeouido) exceeds 25,000 persons/km². Denser deployment at 200m cell radius is recommended.

2.1a Dual Scenario: 48-Core Expansion

Expanding the RU-embedded MOCT ribbon to 48 cores enables two deployment scenarios within the same 400K-pole infrastructure.

Option A — Minimum Deployment (capacity basis): 12M subscriptions ÷ 48 cores = 250,000 RUs (1 per pole, 62.5% pole utilization). 150,000 unused poles reserved for future expansion.

Option B — Full Deployment (pole basis): 400,000 poles × avg. 2.25 directional antennas = 900,000 RUs × 48 cores = 43.2M cores. 3.6× headroom over current 12M subscriptions. Optimizes per-direction coverage, fully absorbs IoT·V2X·PAI future demand. This document's design baseline.

※ This document adopts Option B (full deployment) as the design baseline. Option A may be applied as a phased approach following Phase 1 build-out.

## 2.2 RU Deployment Mode

RUs are directional antennas installed 1–3 per pole, design average 2.25/pole. Each RU covers a distinct directional sector, sharing 360° coverage. RUs on the same pole connect to Face 1 (primary) and Face 6 (extended) of the same H6. Existing 400K utility poles are prioritized to minimize new civil works. Duplicate coverage in the same direction is prohibited — adjacent H6 covers that sector. Each RU ships factory-fused with a MOCT 48-core ribbon cable (7m) as an integrated unit.

# 3. H6 Node Count and Pole Deployment

## 3.1 H6 Count Calculation

One H6 node is installed per pole alongside the RU(s). Each pole holds 1–3 RUs (avg. 2.25); a single H6 accommodates all via Face 1 (primary RU) + Face 6 (additional RUs, max 2). H6 nodes serving wired T4 are counted separately (installed in buildings/complexes). Installation completes with a single field fusion splice connecting the RU-embedded MOCT ribbon (48 cores, 7m) end to the H6 ribbon end. No connectors or patch panels required.

| Calculation Item | Formula / Value | Result |
| --- | --- | --- |
| Total RU Count | Pole basis | 900,000 |
| H6 (RAN pole): 1 per pole, fixed | 1 H6 per pole (accommodates 1–3 RUs) | 400,000 |
| Additional for contingency and mesh (+10%) | 400,000 × 1.10 | 440,000 |
| Planned deployment count (RAN H6) | — | 440,000 |
| H6 (wired T4) | 1 + ceil((5,200,000-4)÷8) | approx. 650,000 |
| Total H6 Count | 440,000 + 650,000 | approx. 1,090,000 |

※ H6 (RAN pole) count of 440,000 = 400,000 poles + 10% contingency (40,000).

※ H6 (wired T4) count of 650,000: derived from 5.2M T4 nodes at max 8 T4 per H6.

## 3.2 H6 Installation Position and Cable Distances

RU and H6 are co-located vertically on the same pole in the following structure:

[ RU — pole top, 5–8m ]

Embedded MOCT ribbon 48 cores · 7m · factory-fused integral unit

│

↓ ← 1 field fusion splice (RU ribbon end ↔ H6 ribbon end)

│

[ H6 — pole base, 1–2m ] ← H6 ribbon cable origin (no connectors)

│  Face 2 → O8 uplink · Face 3/4/5 → adjacent H6 horizontal mesh

↓

[ Optical backbone → O8 ]

※ Field procedure: mount RU on pole → 1 ribbon fusion splice → complete. No connectors, patch panels, or splice enclosures required.

| Segment | Distance | Note |
| --- | --- | --- |
| RU (pole top) → H6 (pole base) | 5 ~ 10 m | MOCT ribbon cable (RU-embedded 7m · 1 field splice) |
| H6 → adjacent H6 (horizontal mesh) | 200 ~ 400 m | Underground or aerial optical cable |
| H6 → O8 (uplink) | 300 m ~ 2 km | Zone aggregation optical cable |
| O8 → D12 (metro backhaul) | 2 ~ 10 km | Underground trunk optical cable |

# 4. MOCT Ribbon Optical Cable — Quantities and Volumes

## 4.1 MOCT Cable Segment Definitions

The MOCT (Multi-Optical-Core Transceiver) ribbon cable is a connector-free, factory-fused multi-core optical cable as defined in Velsanet White Paper #03. Cable segments span 5 defined sections from T4 endpoint to O8. Segment A connects the RU-embedded MOCT ribbon (48 cores, 7m, factory-fused) to the H6 ribbon end with a single field fusion splice.

## 4.4 Segment A — RU↔H6 Vertical Cable

| Item | Calculation | Result |
| --- | --- | --- |
| Total RU Count | Pole basis | 900,000 |
| Cables per RU | 1 per RU | — |
| Total Cable Count | 900,000 × 1 | 900,000 lengths |
| Unit Cable Length (with margin) | Average 7m (+40%) | 10 m each |
| Total Cable Length | 900,000 × 10 m | 9,000 km |

※ As RU count increased from 400,000 to 900,000 (400,000 poles × avg. 2.25), Seg A cable was recalculated from 4,000km to 9,000km. Each RU-embedded MOCT ribbon is 48 cores = 12-core ribbon × 4 bundles, requiring 4 field fusion splices. No connectors or patch panels needed.

4.4a Ribbon Fusion Splice — Technical Specifications

48-Core Structure Definition

RU-embedded MOCT 48 cores = 12-core ribbon × 4 bundles. Field splicing: 4 operations per 12-core unit. Both RU side and H6 side ship with identical 12-core ribbon structure.

Key Characteristics of Ribbon Fusion Splicer

| Item | Description |
| --- | --- |
| Multi-fiber simultaneous splicing | Splices a 12-core ribbon simultaneously via arc discharge. Reduces splice time to 1/12 of single-core splicer. |
| Fixed V-groove method | 12 precision-machined V-grooves auto-align core pitch. Eliminates manual alignment error. |
| Ribbon dedicated holder | Dedicated holder keeps the 12-core ribbon intact when seated in the splicer. Maintaining ribbon form is critical. |

Required Auxiliary Equipment

| Equipment | Function and Requirement |
| --- | --- |
| Thermal Coating Stripper | Removes 12-core coating simultaneously via heat. Standard strippers scatter cores — dedicated tool required. |
| Ribbon Fiber Cleaver | Cleaves all 12 cores at equal length and angle simultaneously. End-face quality critical to minimize splice loss. |
| Ribbon Protection Sleeve | Wide heat-shrink sleeve for splice protection. Must be pre-threaded onto cable before splicing — cannot be inserted post-splice. |

4.4b Field Fusion Splice Procedure — Full 48-Core Connection

Repeat the following procedure once per 12-core ribbon bundle (4 times total) to complete full 48-core connection. The same pre-processing applies to both RU and H6 sides.

| Step | Task | Detail |
| --- | --- | --- |
| 1 | Preparation | Fix one 12-core ribbon bundle from RU in dedicated holder. Pre-thread protection sleeve onto cable (cannot be inserted after splicing). |
| 2 | Pre-processing | Strip 12-core coating with thermal stripper → clean with alcohol → cleave end-face with ribbon cleaver. Apply to both RU and H6 sides. |
| 3 | Splicing | Seat both RU and H6 ribbon ends in splicer V-groove → press once → 12-core simultaneous arc fusion complete. |
| 4 | Protection | Slide ribbon heat-shrink sleeve over splice → heat. Secure in splice enclosure. |
| 5 | Repeat / Complete | Repeat steps 1–4 for remaining 3 bundles (ribbons 2, 3, 4) → 4 total splices = full 48-core connection per RU. |

※ Seoul total: 900,000 RUs × 4 splices = 3,600,000 field splices. At 200 splices/machine/day, approx. 18,000 machine-days required.

4.4c Optical-Wireless Speed Matching and Clock Synchronization Design

The core of Velsanet RU design is precise speed alignment between the wireless signal bandwidth and the total transmission capacity of 8 channels per optical core. A single optical core potentially carries terabit-level (Tbps) bandwidth; when divided into 8 slots, each slot must losslessly accommodate one wireless MIMO stream to achieve 6G ultra-low-latency and ultra-high-speed performance. Clock synchronization across 8 channels is equally decisive — Velsanet resolves both challenges with a single optical core.

1. Optical-Wireless Speed Mapping Structure (Speed Alignment)

| Radio Environment | Optical Core Mapping Strategy | Result |
| --- | --- | --- |
| Low-band (Sub-6GHz) | Aggregate 8+ channels into 1 core | Optical resource efficiency · increased user capacity |
| High-band (mmWave · 6G) | 8-channel MIMO → 1 core 1:1 mapping | Peak 1 Tbps (125 Gbps/channel × 8) |
| Mixed band (incl. THz) | Variable bandwidth allocation (Flexible BW) | Optimized transmission efficiency · future THz band ready |

2. Speed Matching Hardware Components

| Component | Function and Design Significance |
| --- | --- |
| Ultra-high-speed DSP | Real-time framing and compression of 8-channel wireless sampling data to match optical core speed. Prevents radio bandwidth from exceeding optical rate. |
| Coherent Optical Engine | Optical engine using both phase and amplitude to pack 8 channels into 1 core via high-order modulation (PAM4 etc.). Multiplies capacity vs. simple on/off switching. |
| Time-Sensitive Gearbox | Synchronizes data flow across the wireless-optical interface. Hardware logic that minimizes latency between two domains running at different speeds. |

3. Clock Synchronization — Optical Injection Locking

Velsanet dedicates 1 of 48 ribbon cores to clock, transmitting a reference clock from H6 to RU via optical carrier. The RU's Coherent Optical Engine locks onto this signal to synchronize the 8-channel DSP — eliminating separate clock wiring and PTP packets. The fixed physical path makes propagation delay deterministic and correction trivial.

| Method | Legacy (PTP/IEEE 1588) | Velsanet (Optical Injection Locking) |
| --- | --- | --- |
| Clock Distribution Path | Network → PTP packets → clock recovery | H6 → 1 optical core → direct RU lock |
| Latency / Jitter | Packet processing delay and jitter | Physical propagation delay only · deterministic |
| Additional Wiring | Separate clock wiring or GPS antenna required | Not required — 1 of 48 cores allocated |
| H6 Role | Intermediate aggregation node | Aggregation + physical origin of Time Reference |

4. Consistency with Velsanet Structural Simplification Principles

| Eliminated Element | Legacy Approach | Velsanet |
| --- | --- | --- |
| Coaxial cable | RF signal → coaxial transmission | Direct optical core (MOCT integrated) |
| Connectors / Patch panels | Multiple field connection tasks | Factory splice + 4 field ribbon splices |
| Clock wiring | PTP packets / separate clock line / GPS | 1 optical core → optical injection locking |
| DU/CU software layer | Virtualization stack present | Structurally eliminated (physical path direct) |

※ Path to 1 Tbps — LG Electronics demonstrated 320m outdoor THz-band (155–175 GHz) wireless transmission at Fraunhofer Institute, Berlin in 2022, presenting potential 1 Tbps capacity. Velsanet RU achieves equivalent 1 Tbps through MIMO 8-channel parallel transmission in lower mmWave bands — offering superior coverage stability vs. high-frequency single-channel approaches while maintaining continuity with existing mmWave infrastructure. The more practical path to 1 Tbps for city-wide Seoul deployment.

Velsanet’s design achieves 6G’s 1 Tbps target while maintaining the infrastructure completeness to deploy across all of Seoul through zero-coax and direct MOCT connection.

## 4.5 Segment B — H6↔H6 Horizontal Mesh Cable

| Item | Calculation | Result |
| --- | --- | --- |
| Total H6 Count (RAN) | Pole basis | 440,000 |
| Avg. mesh connections per H6 | 2.5 links (Face 3/4/5) | — |
| Total links (bidirectional deduplicated) | 440,000 × 2.5 ÷ 2 | 550,000 links |
| Unit Cable Length (with margin) | Average 300m × 1.20 | 360 m/link |
| Total Cable Length | 550,000 × 360 m | 198,000 km |

※ H6 count increase (1,550 → 440,000) expands Seg B total from 700km to 198,000km. Sharing existing KT/SKT underground conduit can reduce new civil construction costs by approx. 30%.

## 4.6 Segment C — H6→O8 Uplink Aggregation Cable

| Item | Calculation | Result |
| --- | --- | --- |
| Total H6 Count (RAN) | 440,000 | — |
| Cables per H6 (Face 2) | 1 per H6 | — |
| Total links | 440,000 lengths | 440,000 lengths |
| Unit Cable Length (with margin) | Average 800m × 1.25 | 1,000 m/link |
| Total Cable Length | 440,000 × 1,000 m | 440,000 km |

※ H6 count increase recalculates Seg C total from 1,550km to 440,000km.

## 4.6b Segment D — O8↔O8 Horizontal Mesh Links

O8 nodes connect directly to adjacent O8 nodes via horizontal faces (3–4 per node). This horizontal mesh provides per-user E2E path diversity and failure bypass routes, forming the foundation for autonomous traffic distribution at the O8 layer.

| Item | Calculation | Result |
| --- | --- | --- |
| Total O8 Count | Default design value | 2,521 |
| Horizontal connections per O8 (Face 3–4 basis) | Average 3.5 (midpoint of 3–4) | — |
| Total links (bidirectional deduplicated) | 2,521 × 3.5 ÷ 2 | ≈ 4,412 links |
| Unit Link Distance (with margin) | Average 750m × 1.20 | 900 m/link |
| Total Cable Length | 4,412 × 900 m | ≈ 3,971 km |

※ O8↔O8 horizontal links utilize Seoul's underground trunk optical trenches. Average inter-O8 distance 700–800m; 20% margin applied → 900m/link. 4,412 links ensure full mesh connectivity across Seoul's O8 layer.

## 4.7 Total Optical Cable Summary

| Segment | Purpose | Count | Length (km) | Note |
| --- | --- | --- | --- | --- |
| Seg T0 | T4 ↔ T4 intra-building direct | approx. 1.11M | 38,850 | Using existing conduit |
| Seg T1 | T4 → H6 Face building ingress | approx. 5.2M | 99,840 | Average 19m |
| Seg A | RU → H6 vertical | 900,000 | 9,000 | RU-embedded 48-core · 10m · 4 field splices |
| Seg B | H6 ↔ H6 horizontal mesh | 550,000 | 198,000 | Underground buried |
| Seg C | H6 → O8 uplink aggregation | 440,000 | 440,000 | Trunk optical cable |
| Total | All segments | approx. 6.32M | 780,690 km | Infrastructure + T4 included |

※ Seg A·B·C (infrastructure segments) total: 647,000km. T0·T1 (T4 access segments) total: 138,690km. Grand total approx. 785,690km.

※ Seg T0·T1 leverage existing in-building wiring infrastructure — no new civil works required.

# 6. O8 Node — Face Structure and Count

The O8 (Octahedron) node has 8 faces. Each face is allocated to uplink (D12), downlink (H6), or horizontal (O8↔O8 mesh) functions. Maximum O8 cores = 192 × 8 = 1,536; H6 capacity per O8 is determined by the number of downlink faces.

## 6.1 O8 Face Function Allocation

| Direction | Faces | Core Count | Note |
| --- | --- | --- | --- |
| Uplink (D12) | 1 ~ 2 | 192 ~ 384 | Default 1, dual for redundancy |
| Downlink (H6 aggregation) | 1 ~ 2 | 192 ~ 384 | Default 1, 2 for high-density zones |
| Horizontal (O8↔O8 mesh) | 3 ~ 4 | 576 ~ 768 | Urban mesh connectivity |
| Total | 8 face | 192 × 8 = 1,536 | O8 max cores |

※ Max cores per O8: 192 × 8 = 1,536. H6 capacity with 1 downlink face: 192. With 2 downlink faces (high-density): 384.

## 6.2 O8 Count Calculation — Full H6 Coverage Basis

| Item | Calculation | Result |
| --- | --- | --- |
| Total H6 (RAN) | Pole basis +10% | 440,000 |
| O8 downlink faces (default) | 1 face | — |
| H6 capacity per O8 (default) | 1 × 192 | 192 H6/O8 |
| Required O8 count (default) | 440,000 ÷ 192 | ≈ 2,292 |
| Contingency +10% | 2,292 × 1.10 | ≈ 2,521 |
| High-density zone (2 downlink faces) | 2 × 192 = 384 H6/O8 | Optional expansion |

※ Default design value: 2,521 O8 nodes (1 downlink face · 192 H6/O8).

※ High-density zones (urban core etc.): applying 2 downlink faces allows 384 H6 per O8 → halves the required O8 count.

※ The previous O8 count of 285 was calculated against 1,550 H6 nodes and was incorrect. The corrected value based on 400K poles is 2,521.

## 6.3 O8 Deployment Scenarios — Urban Intelligence Density

The baseline O8 count of 2,521 is derived from H6 aggregation capacity: 440,000 H6 ÷ 192 H6/O8 × 1.1 = 2,521. At this density, each O8 covers an average area of 605 km² ÷ 2,521 ≈ 0.24 km² — a square with ~490m sides, consistent with the ~390m radius referenced in this document.

However, this density is adequate for human-scale connectivity — not for a future urban environment where O8 operates as the origination point for distributed AI collaboration and urban traffic intelligence. When the design basis shifts from “how many H6 nodes to aggregate” (communication density) to “how many concurrent agents and events to handle” (urban intelligence density), the appropriate O8 count must be redefined accordingly.

The O8 count is not a question of communication density — it is a question of urban intelligence density.

| Scenario | O8 Count | Design Basis |
| --- | --- | --- |
| Baseline | 2,521 | Minimum aggregation structure per current H6 design. ~0.24 km²/O8 · ~490m cell side. This document's baseline value. |
| Expanded | 5,000+ | High-density robotics · V2X · CCTV · real-time control. O8 operated as active event-processing grid node, not passive aggregator. |
| Urban AI | 7,000 – 10,000+ | Autonomous systems · public control · large-scale multi-agent coordination. O8 as the fundamental unit of the urban intelligence grid. |

※ 2,521 O8 nodes is the minimum baseline to accommodate Seoul's current H6 aggregation structure. In a future urban environment including robotics, autonomous mobility, CCTV, public sensing, and AI-native machine coordination, the practical O8 count must expand based on control density, event concurrency, and distributed intelligence requirements. The final O8 count is not fixed — it scales with the urban intelligence density the city chooses to deploy.

# 8. Seoul Deployment Final Summary (Rev.1)

| Component | Count | Note |
| --- | --- | --- |
| RU (Radio Unit) | 900,000 | Directional antenna 1–3/pole · avg. 2.25 · 6G mmWave · 48 cores embedded |
| H6 Node (RAN pole) | 440,000 | Pole base · 10% contingency included |
| H6 Node (wired T4) | approx. 650,000 | Installed in buildings / complexes |
| Total H6 Count | approx. 1,090,000 | RAN + wired combined |
| T4 Node (wired) | 5.2M | Households · offices · IoT |
| Optical Cable Infrastructure (Seg A+B+C) | 647,000 km | A:9,000 + B:198,000 + C:440,000 |
| Total Optical Cable (incl. T4) | approx. 780,690 km | T0+T1+Infrastructure combined |
| O8 Node (default) | 2,521 | 1 downlink face · ~390m radius |
| O8 Node (high-density expansion) | 2 downlink faces applied | 192×2=384 H6/O8 · optional |

## 8.1 Layer-by-Layer E2E Flow Summary

| Layer | Node | Count | Role |
| --- | --- | --- | --- |
| L0 | T4 | 5.2M | Subscriber endpoint |
| L1a | H6 (RAN pole) | 440,000 | RAN + wired convergence |
| L1b | H6 (wired T4) | approx. 650,000 | Building / complex wired aggregation |
| L2 | RU | 900,000 | 6G wireless RAN · 48 cores/RU · directional antenna |
| L3 | O8 | 2,521 (default) | Intelligence · E2E management · 192×8 cores |
| L4 | D12 | TBD (subsequent document) | Metro/WAN · AAI |

## 8.2 Comparison — Legacy 5G vs. Velsanet 6G (Seoul Basis)

| Item | Legacy 5G (estimated) | Velsanet 6G |
| --- | --- | --- |
| Base Station Count | approx. 12,000 (macro + small cell) | 900,000 RUs (400,000 poles×2.25) + 440,000 H6 (RAN) |
| DU / CU Equipment | Required (software layer) | Not required (structurally eliminated) |
| E2E Path Method | Packet switching + session management | Physical optical core 1:1 dedicated |
| Handover | Re-authentication required · potential packet loss | Overlapped E2E · lossless |
| Wired Integration | Separate FTTx network required | T4→H6 unified single network |
| O8 Intermediate aggregation node | N/A | 2,521 nodes · 192×8 cores · AI-integrated |

※ Node counts for layers D12 and above (metro backhaul, national network) are defined in a separate document (Velsanet-DEPLOY-NAT-01).

Velsanet Architecture Group · Velsanet Research Institute (Proposed) · Republic of Korea

github.com/joa337/velsanet-whitepapers · 2026

# Appendix – CAPEX Estimation

Velsanet Structure-Native Network Architecture

Seoul Metropolitan 6G Infrastructure Deployment Plan

## A.1 Purpose

This appendix provides an indicative CAPEX estimation for deploying the Velsanet Structure-Native network architecture across Seoul. Estimates are based on confirmed node counts (RU, H6, O8, T4) and total optical cable length, applying industry-average unit cost assumptions. Actual costs will vary based on equipment design, supply chain, construction methods, and reuse of existing infrastructure.

## A.2 Seoul Infrastructure Deployment Baseline

| Component | Count | Note |
| --- | --- | --- |
| T4 Node | 5,200,000 | Household / office wired access endpoint |
| RU (Radio Unit) | 900,000 | Pole-based 6G mmWave wireless access |
| H6 Node (RAN pole) | 440,000 | RU aggregation and optical branching |
| H6 Node (wired T4) | 650,000 | Building / complex wired aggregation |
| Total H6 Nodes | approx. 1,090,000 | RAN + wired combined |
| O8 Node | 2,521 | Urban aggregation and E2E management layer |
| Total Optical Cable Length | approx. 785,690 km | Seg A–D + T0 + T1 combined (Rev.9 basis) |

## A.3 Equipment Unit Cost Assumptions

※ Unit costs assume industry-average pricing following mass production and standardization.

| Equipment | Unit Cost Assumption | Note |
| --- | --- | --- |
| T4 Node | $1,480 (KRW 2M) | Single unit price |
| H6 Node | $5,930 (KRW 8M) | Single unit price |
| RU (Radio Unit) | $2,200–$5,930 (KRW 3M–8M) | Low / Mid / High scenario |
| O8 Node | $14,800–$22,200 (KRW 20M–30M) | Low / High scenario |

## A.4 Seoul Velsanet Total CAPEX Scenarios

| Item | Existing Infrastructure Reuse | Mixed Build | Greenfield Build |
| --- | --- | --- | --- |
| Equipment CAPEX | approx. $16B–$19B | approx. $16B–$19B | approx. $16B–$19B |
| Optical Infrastructure Cost (per km) | $7,400/km → approx. $5.8B | $14,800/km → approx. $11.6B | $37,000/km → approx. $29.1B |
| Total CAPEX Estimate | approx. $22B–$25B | approx. $28B–$31B | approx. $45B–$48B |

## A.5 Structural Significance

Velsanet infrastructure costs represent more than mobile network investment. This architecture consolidates mobile networks, wired access networks, IoT connectivity, V2X communication infrastructure, and AI/robotics connectivity into a single structural optical infrastructure.

This architecture replaces multiple legacy networks (FTTx, mobile backhaul, metro rings, aggregation layers) with a unified optical infrastructure.

Velsanet is best understood not as a telecommunications investment but as a city-scale digital infrastructure redesign project — restructuring Seoul's fragmented legacy infrastructure into a single structural network.

Velsanet Architecture Group · Republic of Korea · 2026